Use at your own risk. The pilot must refer to the FAA approved documents for the correct information, and is responsible for safe operation of aircraft.

| Cessna 172M – 160HP V1.8 12/3/2025 |  |
| --- | --- |
| TAKEOFF Normal: Rotate 60 mph Vy 91 mph (80 mph @ 10,000’ DA) Short/Soft: Flaps 0 /10° Vx 68 mph (73 mph @ 10,000’ DA) |  |
| CRUISE CLIMB 80-90 mph @ Max RPM Do Not exceed 2500 RPM for more than 5 min during climb out BEST GLIDE 80 mph (flaps up) |  |
| >1000’ AGL Oil Pres/Temp Ammeter Lights |  |
| CRUISE Do Not exceed 2500 RPM for more than 5 min during Cruise Mixture LEAN for max RPM Heading Indicator SET Routinely Check Gauges: Oil Press/Temp Ammeter Fuel Carb Temp (64R) |  |
| LANDING Landing Light ON Carb Heat ON Fuel BOTH Mixture ENRICH Approach at 70-80 mph (flaps up) 65-75 mph (flaps down) |  |
| GO AROUND Full Throttle Pitch up Carb Heat OFF Flaps retract to 20° Climbing & above 65 mph retract flaps |  |
| AFTER LANDING Carb Heat OFF Flaps UP Mixture LEAN for taxi |  |
| SHUTDOWN Avionics/Lights OFF Mixture OFF Master OFF Mags OFF Parking Brake SET Pitot Tube Cover Installed Control Lock Installed Flight Plan Closed |  |
| POSTFLIGHT Hobbs & Tach Time RECORD Windows & Leading Edges CLEAN Wheel Chocks SET Seat Belts SECURE Interior CLEANED Airplane LOCKED Flight Log ENTERED Flight Plan CLOSED Hangar SECURE Lights OFF Doors LOCKED |  |
| IAS MPH Vso 54 Vs1 61 Vfe 100 Va 2300 112 Va 1950 103 Va 1600 93 Vno 145 Vne 182 | Recommended Pattern Speeds/ Approx Pwr Settings Downwind: 90 - 95 mph / 2000 RPM Abeam: 80 - 85 mph / 1500 RPM Base: 75 - 80 mph / 1500 RPM Final: 65 - 70 mph / 1500 RPM (Flaps – No Flaps) |
| Emergency: Com 121.5 / Squawk 7700 Loss Com. Squawk 7600 KMAN ASOS:118.325 KEUL ASOS :135.07 KBOI ATIS:123.9 BOI App/Dep:119.6 unless directed to 126.9 BOI Twr 118.1 / Gnd 121.7 FSS BOI: 122.6 |  |

Use at your own risk. The pilot must refer to the FAA approved documents for the correct information, and is responsible for safe operation of aircraft.

| Cessna 172M – 160HP V1.8 12/3/2025 |  |
| --- | --- |
| ## PREFLIGHT Pitot/AOA Tube Covers-REMOVED Gust Lock - REMOVED Avionics Master–OFF AOA Pitot Heat/Pitot Heat - OFF Master – ON (Right side only) Hobbs – Check with Flight Log Tach Time Check < Oil Change Flaps – 20° Smooth Operation Fuel Gauges – CHECK Exterior Lights - CHECK Interior Lights - CHECK Master – OFF Baggage Door – LOCKED Tires/Brakes – Press/Leaks (Main 29 psi - Nose 31 psi) Fuel Quantity & Caps Secure. Oil level: 6 MIN / 7 MAX Qts. Nose Strut - Correct travel Prop/Spinner - Inspect Static Ports - Clear Check Fuel Wing Drains - Quality Engine Fuel Strainer-Drain Final Walk/Look Around/Tow Bar ## START Seats – LOCKED Seat/Shoulder Belts– FASTENED Parking Brake – SET Passenger Briefing Fuel – ON BOTH Circuit Breakers – IN Mixture – RICH Carb Heat – COLD Prime – As Required (max 3) Throttle – OPEN ½ Inch Beacon – ON Prop Area - Clear (Yell) Master – ON (Right side only) Engage Starter (max 20 sec crank if no start) | ## AFTER START Master – ON (Left and Right) Throttle – 1000 RPM Oil Pressure– in green < 30 seconds Flaps – UP Mixture – LEAN FOR TAXI Avionics Master – ON GTN750Xi – Set Fuel Qty/Fuel Burn Taxi Lights - ON Transponder – ALT ## RUNUP Doors & Windows – LATCHED Controls – FREE & CORRECT Flaps – UP Trim – TAKEOFF Fuel – BOTH Throttle – 1700 RPM Mixture – FULL RICH (Lean for Best Power above 3,000’ DA) Mags (drop< 125, difference ea <50) Carb Heat – TEST Ammeter - Charging Low Voltage Light - Out Throttle – back to 1000 RPM Instruments – CHECK/SET Garmin G5 baro pressure set/referenced same as conventional Altimeter Seat Belts/Harness – SECURE COM – First Freq. 122.7 KMAN KMAN ASOS:118.325 Headsets Check–Noise Canceling ON Check Intercom/Com volumes/operation GTN750Xi/Avionics – Set Up |
| BEFORE TAKEOFF Flaps UP OR 10° Carb Heat OFF Landing Lights/Strobes ON Headset ON Transponder - ALT - Clearing circle for Pattern Traffic What is your ABORT Plan? |  |